MSW-11.42

5” Bass Midrange

| SPECIFICATIONS |  |  |  |
| --- | --- | --- | --- |
| Nominal Power Handling (RMS) |  | P | 150 W |
| Transient Power - 10 ms |  |  | 1,000 W |
| Nominal Impedance |  | Z | 8  |
| Sensitivity 1W/1M |  |  | 86 dB |
| Frequency Response |  |  | 40 – 5,000 Hz |
| Resonant Frequency |  | FS | 45 Hz |
| VOICE COIL |  |  |  |
| Voice Coil Diameter |  |  | 75mm (3.0”) |
| Voice Coil Height |  |  | 12 mm (0.47”) |
| Voice Coil Former |  |  | aluminum |
| Voice Coil Wire |  |  | hexagonal shaped aluminum |
| Number of Layers |  |  | 2 |
| DC Resistance |  | RE | 5.2  |
| Voice Coil Inductance @ 1 kHz |  | LBM | 0.38 mH |
| MAGNET SYSTEM |  |  |  |
| Magnet System Type |  |  | double ferrite, vented |
| Magnetic Gap Height |  | HE | 5 mm (0.2”) |
| Flux Density |  | B | 0.62 T |
| BL Product |  | BXL | 5.90 NA |
| Max. Linear Excursion |  | X | 3.5 mm (0.14”) |
| OPERATIONAL PARAMETERS |  |  |  |
| Suspension Compliance |  | CMS | 1,023 M / newton |
| Mechanical Q Factor |  | QMS | 1.80 |
| Electrical Q Factor |  | QES | 0.51 |
| Total Q Factor |  | Q/T | 0.40 |
| Mechanical Resistance |  | RMS | 1.915 Kg S-1 |
| Moving Mass |  | MMS | 12.2 g |
| Equivalent Cas Air Load |  | VAS | 11.70 L |
| Cone / Dome Material |  |  | damped polymer composite |
| Effective Piston Area |  | S | 90 cm2 |
| Dimensions | Diameter Cutout Height Mounting Depth |  | 142mm (5.60”) 121mm (4.76”) 61mm (2.40”) 56mm (2.20”) |
| Net Weight |  |  | 0.97 kg (2.1 lbs) |