MST-205

Soft Dome Tweeter

| SPECIFICATIONS |  |  |  |
| --- | --- | --- | --- |
| Nominal Power Handling (RMS) |  | P | 120 W |
| Transient Power - 10 ms |  |  | 500 W |
| Nominal Impedance |  | Z | 8  |
| Sensitivity 1W/1M |  |  | 90 dB |
| Frequency Response |  |  | 1,600 – 25,000 Hz |
| Resonant Frequency |  | FS | 650 Hz |
| VOICE COIL |  |  |  |
| Voice Coil Diameter |  |  | 28 mm (1.1”) |
| Voice Coil Height |  |  | 2.5 mm (0.1”) |
| Voice Coil Former |  |  | aluminum |
| Voice Coil Wire |  |  | copper |
| Number of Layers |  |  | 2 |
| DC Resistance |  | RE | 5.2  |
| Voice Coil Inductance @ 1 kHz |  | LBM | 0.05 mH |
| MAGNET SYSTEM |  |  |  |
| Magnet System Type |  |  | ferrite, vented pole |
| Magnetic Gap Height |  | HE | 2.5 mm (0.1”) |
| Flux Density |  | B | 1.45 T |
| BL Product |  | BXL | 3.5 NA |
| Max. Linear Excursion |  | X | - |
| OPERATIONAL PARAMETERS |  |  |  |
| Suspension Compliance |  | CMS | - |
| Mechanical Q Factor |  | QMS | - |
| Electrical Q Factor |  | QES | - |
| Total Q Factor |  | Q/T | - |
| Mechanical Resistance |  | RMS | - |
| Moving Mass |  | MMS | 0.50 g |
| Equivalent Cas Air Load |  | VAS | - |
| Cone / Dome Material |  |  | selected soft fabric |
| Effective Piston Area |  | S | 6.0 cm2 |
| Dimensions | Faceplate Diameter Faceplate Thickness Cutout Mounting Depth |  | 104mm (4.09”) 3.5mm (0.14”) 74mm (2.91”) 43mm (1.69”) |
| Net Weight |  |  | 0.55 kg (1.2 lbs) |